Contents

1	ArduCAM + Arduino Mini D1	1

1.1	Installing libraries	1

1.1.1	Time	1

1.1.2	ArduCAM	1

1.1.3	ArduinoJson	1

1.2	Installing OV7670 WITH ARDUCAM REV C+ camera shield with SERIAL DATA interface to Wemos D1 mini	1

1.2.1	ArduCAM Rev C+ Pin connections:	1

1.2.2	Hardware Circuit Connection btwn D1 and Arducam	3

1.2.3	SW setup	3

1.2.4	Wiring OV7670 w/o FIFO to the Arducam board	3

1.2.5	Basic “is it alive” setup	5

1.2.6	Wiring OV7670 FIFO	6

1.2.7	Tuning	7

1.2.8	Executing code	7

1.2.9	Alternative solutions:	7

1.2.10	Converting RAW to image	7

2	Set up Arduino for camera	7

2.1	Tech refs:	7

2.2	Installing Uno R3 hardware and run “Blink”	7

2.3	Installing OV7670 NO FIFO camera	7

2.3.1	Wiring OV7670 NO FIFO	7

2.3.2	Tuning	7

2.3.3	Executing code	7

2.3.4	Alternative solutions:	8

2.3.5	Converting RAW to image	8

2.4	Installing OV7670 WITH FIFO (=AL422) camera	8

2.4.1	Basic “is it alive” setup	8

2.4.2	Wiring OV7670 FIFO	8

2.4.3	Tuning	9

2.4.4	Executing code	9

2.4.5	Alternative solutions:	9

2.4.6	Converting RAW to image	10

2.5	Installing wifi shield ESP-01 ESP-8266	10

2.5.1	Wiring	10

2.5.2	Booting	10

2.5.3	Flashing ESP	11

3	ESP8266 Arduino board (ESP12-E Arduino = D1)	13

3.1	Install drivers and sw	13

3.1.1	Install board on Arduino (I have a D1, not a D1R1):	13

4	Barcode reader programming	13

4.1	Test programs for development	13

4.1.1	29/11-16: sending messages to IOT hub from Arduino using Visual Studio:	13

4.2	Sending a file over wifi	13

5	Visual Studio setup for Arduino	14

5.1.1	Running and debugging	14

5.1.2	Install the right VS extensions :	14

5.1.3	Important note	14

5.1.4	In case of errors	14

5.1.5	Path variables	14

5.1.6	Install curl library:	15

6	Java (not used)	16

ArduCAM + Arduino Mini D1

Installing libraries

You need the following libraries into the Arduino library. Find this from the Arduino IDE => File => Settings. 
The correct folder is <sketchbook location>/libraries. 
On Dell PC: J:\NNR-DATA\_Dev\src\Arduino\libraries

NOTE: If you want to MODIFY the libraries, they must be installed in a user library folder.
On Dell-work PC, this is in C:\data_no_backup\DevelopmentPriv\Arduino\libraries
On Dell-work PC, the regular library folder is in C:\Program Files (x86)\Arduino\libraries

When done, it looks like this:

Time

- Download and install from ZIP using Visual Studio

ArduCAM

- Download from https://github.com/ArduCAM/Arduino

- Copy the two directories ArduCAM and UTFT4ArduCAM_SPI  (leave the rest) to the Arduino library folder (see above for how to find it). Remember to refresh in Visual Stuio (or restart).

ArduinoJson

Installing OV7670 WITH ARDUCAM REV C+ camera shield with SERIAL DATA interface to Wemos D1 mini

The D1 mini does not have 8 available data inputs, hence we need to connect it using serial data interface.

Links:

http://usemodj.com/2016/08/25/esp8266-arducam-5mp-ov5642-camera-wifi-video-streaming/ (instructions for this build)

http://www.arducam.com/downloads/shields/ArduCAM_RevC+_Camera_Shield_DS.pdf (user manual for Arducam shield)

http://www.arducam.com/arducam-rev-c-plus-shield-released/#more-991

http://www.arducam.com/category/introduction/ (I have not used the hardware – only the SW)

https://www.wemos.cc/product/d1-mini.html (Wemos D1 Mini reference)

Hardware note: UC-260 camera module = OV2640 2Mp camera

Follow instructions on http://usemodj.com/2016/08/25/esp8266-arducam-5mp-ov5642-camera-wifi-video-streaming/

ArduCAM Rev C+ Pin connections:

http://www.arducam.com/downloads/shields/ArduCAM_RevC+_Camera_Shield_DS.pdf

Figur 1. Arducam REV C+ pin layout

4: VCC

2: SCK

5: MOSI

6: GND

1: RESET

3: MISO

Figur 2. ICSP (=SPI port) seen from front of Arducam = as in picture above

Wemos D1 MINI

| Pin | Function | ESP-8266 Pin |
| --- | --- | --- |
| TX | TXD | TXD |
| RX | RXD | RXD |
| A0 | Analog input, max 3.3V input | A0 |
| D0 | IO | GPIO16 |
| D1 | IO, SCL | GPIO5 |
| D2 | IO, SDA | GPIO4 |
| D3 | IO, 10k Pull-up | GPIO0 |
| D4 | IO, 10k Pull-up, BUILTIN_LED | GPIO2 |
| D5 | IO, SCK | GPIO14 |
| D6 | IO, MISO | GPIO12 |
| D7 | IO, MOSI | GPIO13 |
| D8 | IO, 10k Pull-down, SS | GPIO15 |
| G | Ground | GND |
| 5V | 5V | - |
| 3V3 | 3.3V | 3.3V |
| RST | Reset | RST |

Figur 3. Wemos D1 Mini pin layout

Hardware Circuit Connection btwn D1 and Arducam

| Pin function | Arducam Rev C+ pin name | Pin position | Wemos D1 mini name | D1 mini pin position | Arduino UNO name | Pin position | Typical name in code |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CS, SD card | 9 | P1-9 |  |  | 9 | 9 | SD_CS |
| CS, SPI port | 10 | P1-10 | D0 (GPIO 16) | Upper-6 | 10 | 10 | SPI_CS |
| SDA | SDA | P1-16 | D2 (GPIO 4) | Lower-5 | SDA |  |  |
| SCL | SCL | P1-17 | D1 (GPIO 5) | Lower-6 | SCA |  |  |
| RESET | - | SPI-1 |  |  | (SPI port, reset) | SPI-1 |  |
| SCK | SCK | SPI-2 | D5 (GPIO 14) | Upper-5 | (SPI port, SCK) | SPI-2 |  |
| MISO | MISO | SPI-3 | D6 (GPIO 12) | Upper-4 | (SPI port,, MISO | SPI-3 |  |
| 5V | 5V | P3-4 | 5V | Lower-1 | (SPI port, 5V) | SPI-4 |  |
| MOSI | MOSI | SPI-5 | D7 (GPIO 13) | Upper-3 | (SPI port,MOSI) | SPI-5 |  |
| GND | GND | SPI-6 | G (GND) | Lower-2 | (SPI-port gnc) | SPI-6 |  |

SW setup

Modifications from instruction notes (http://usemodj.com/2016/08/25/esp8266-arducam-5mp-ov5642-camera-wifi-video-streaming/):

Section “Required libraries” in instruction notes

Arduino library named arduCAM in C:\Users\Nikolaj\AppData\Local\Arduino15\staging\libraries     ELLER     J:\Programmer\Arduino\libraries\

Modify Memorysaver.h:

- #define ARDUCAM_SHIELD_REVC

- //#define ARDUCAM_SHIELD_V2

- //Step 2: Select one of the camera module, only one at a time

- #if (defined(ARDUCAM_SHIELD_REVC) || defined(ARDUCAM_SHIELD_V2))

- #define OV2640_CAM

Change Wifi AP

- const char* ssid = "TeliaGateway58-98-35-B5-DB-17"; // Put your SSID here

- const char* password = "E5B004E62E"; // Put your PASSWORD here

OLD STUFF FROM HERE ONWARDS:

Wiring OV7670 w/o FIFO to the Arducam board

Figure 1. Arducam Rev C+ pinout (http://www.arducam.com/hardware/)

Arducam Rev C+ pinout

Figure 2. Arducam “7670” cam module pinout (http://www.arducam.com/camera-modules/0-3mp-ov7670/ov7670_3-2/) versus standard 7670 cam module. NOTE: one shot from front, the other from back

Pins match just fine:

| Pin nbr | Pin name, uc-246 | OV7670 pin | OV7670 FIFO pin | Arducam Rev C+ camera interface pins |
| --- | --- | --- | --- | --- |
| 1 | 3V3 | 3V3 | 3V3 | +3.3V |
| 3 | SCL | SIOC | SIOC | SCL |
| 5 | VS | VSYNC | VSYNC | VSYNC |
| 7 | PCK | PCLK | D7 | PCLK |
| 9 | D7 | D7 | D5 | D7 |
| 11 | D5 | D5 | D3 | D5 |
| 13 | D3 | D3 | D1 | D3 |
| 15 | D1 | D1 | RST | D1 |
|  |  |  | STR |  |
|  |  |  | WR |  |
|  |  |  | WRST |  |
| 2 | GND | GND | GND | GND |
| 4 | SDA | SIOD | SIOD | SDA |
| 6 | HS | HREF | HREF | HREF |
| 8 | XCK | XCLK | D6 | XCLK |
| 10 | D6 | D6 | D4 | D6 |
| 12 | D4 | D4 | D2 | D4 |
| 14 | D2 | D2 | D0 | D2 |
| 16 | D0 | D0 | PWDN | D0 |
|  |  |  | RCK |  |
|  |  |  | OE |  |
|  |  |  | RRST |  |

Basic “is it alive” setup

Note: The Arduino WeMOS ESP-12 D1 board uses 3.3V, i.e. pull-ups are not necessary between D1 board and Arducam (which handles both 3.3 and 5V).

| Wemos ESP8266 D1 | Arducam |  |
| --- | --- | --- |
|  | VCC SCK MOSI GND RESET MISO (SPI port seen from front of Arducam = as in picture above) | (for reference only) |
| On-board pin name | On-board pin name (on SPI port) |  |
| 3.3V | VCC |  |
| D12/MISO | MISO |  |
| D11/MOSI | MOSI |  |
| D13/SCK | SCK |  |
| GND | GND |  |
| GND or 3.3V ?? | RESET |  |

Figure 3. Schematic pin config for SPI-port (male pins located on Arduino). Must be mirrored to reflect Arducam.

Sketch supporting this test: OV7670FIFO_simple_communication.ino

Wiring OV7670 FIFO

| ESP8266 D1 |  |  |  |  |  | OV7670 FIFO |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name on board | Pin number (pin1=RX) | PIN pins_arduino.h & common.h | PIN Symbol inherited from pins_arduino.h & common.h in ov7670fifo.c | PIN Symbol value in ov7670fifo.c | PIN Nbr new sw | Name on board | Purpose | In or out |
| 3.3V |  |  |  |  |  | 3.3V |  |  |
| GND |  |  |  |  |  | GND |  |  |
| RX<-D0 | 1 | 3 |  |  |  |  |  |  |
| TX->D1 | 2 |  |  |  |  |  |  |  |
| D2 | 3 |  | DO0 = D2 | 16 |  |  |  |  |
| D15/SCL/D3 | 4 | 5 | DO7 = D3 | 5 | 17 | SIOC | SCCB i/f ctrl clock | IN |
| D14/SDA/D4 | 5 | 4 | DO4 = D4 | 4 | 18 | SIOD | SCCB i/f serial data | IN/OUT |
| D13/SCK/D5 | 6 | 14 | DO3 = D13 DO5 = D5 | 14 (/ 23) | 27 | RRST (blue / old:black) | Read pointer reset terminal |  |
| D12/MISO/D6 | 7 | 12 | DO2 = D12 DO6 = D6 | 12 (/ 25) | 28 | WRST (pink / old:white) | Write pointer reset terminal |  |
| D11/MOSI/D7 | 8 | 13 | DO1 = D11 | 13 |  | n/c |  |  |
| D8 | 9 |  |  |  |  | RCK | FIFO memory read clock control |  |
| TX1/D9 | 10 |  |  |  |  | WR | FIFO write control (1=write, 0=no write) | IN |
| D10/SS | 11 | 15 | SS | 15 |  |  |  |  |
| D11/MOSI | 12 |  | MOSI_PIN | 13 |  |  |  |  |
| D12/MISO | 13 |  | SPI_SCK_PIN | 14 |  |  |  |  |
| D13/SCK | 14 |  |  |  |  |  |  |  |
| GND | 15 |  |  |  |  | OE (green / old:green) | FIFO off control | IN |
| (blank) | 16 |  |  |  |  |  |  |  |
| D14/SCM | 17 |  |  |  |  |  |  |  |
| D15/SCL | 18 |  |  |  |  |  |  |  |
| SS |  | 15 |  |  |  |  |  |  |
|  |  |  |  |  |  | HREF | Line sync | OUT |
|  |  |  |  | 22 |  | VSYNC | Frame sync | OUT |
| GND |  |  |  |  |  | PWDN | Power down | IN |
|  |  |  |  | 35 |  | DO0 |  |  |
| 3.3V |  |  |  | 26 |  | RST | Reset | IN |
| D6 | 7 |  | DO2/DO6 | 12 |  |  |  |  |
| D7 | 8 |  | DO1 | 13 |  |  |  |  |
| D8 |  |  |  |  |  |  |  |  |

The pins on OV7670 are explained here: http://www.electrodragon.com/w/OV7670_Module

Relations to Arduino UNO pins: http://www.instructables.com/id/Programming-the-WeMos-Using-Arduino-SoftwareIDE/?ALLSTEPS

Tuning

Blurry picture: Turn the lens manually until it’s sharp.

Executing code

The COM port is used for communication between PC side and Arduino as follows:

Use this setup:

PC side:

Use ReadSerialPortWin project:

- Start the program

- Select the COM port

- WAIT pressing start until the Arduino program has been loaded

Arduino side

Download the program via the COM port before step 3 above.

Do NOT use Serial.begin, Serial.println etc. Everything written to Serial is (of course) sent to the PC program

DO NOT use the port monitor in Arduino IDE / Visual Studio. This will lock the COM port, and then the PC app cannot read it.

Alternative solutions:

NOTE: There may be need for an external oscillator. However, I suspect that this is only needed if we need video. If that’s the case, have a look at this video: https://www.youtube.com/watch?v=gp0FxbEmRSw

This link seems pretty solid, but also use an oscillator: http://www.arducam.com/camera-modules/0-3mp-ov7670/

Converting RAW to image

Set up Arduino for camera

Tech refs:

Pin layout: https://github.com/Bouni/Arduino-Pinout

Installing Uno R3 hardware and run “Blink”

https://learn.adafruit.com/lesson-0-getting-started/breadboard?view=all

Download exe file and install. The Arduino IDE will find the board itself.

Set correct COM port (On \\Z it’s COM4)

https://learn.adafruit.com/adafruit-arduino-lesson-1-blink/other-things-to-do?view=all

(just to verify that there is hole through.

Installing OV7670 NO FIFO camera

This link states: “if you just want to send data from the camera to your PC using the Arduino you don't need external RAM. Here is the code that will accomplish that”: http://forum.arduino.cc/index.php/topic,159557.0.html

http://forum.arduino.cc/index.php?topic=159557.780 
Post #774 has some interesting pin connections to a wifi shield (not the one I have, though).
Post #754 seems interesting

We will in any case need to convert from 5V to 3.3V, and this is shown in this link.
However, the link has some notes stating that it’s the wrong camera type. In contrast to this, the picture shows the right model, and other notes states that it works great. So give it a try. 
http://www.elecfreaks.com/projects/how-to-use-ov7670-camera-module-with-arduino/

Wiring OV7670 NO FIFO

https://github.com/ComputerNerd/ov7670-no-ram-arduino-uno

Tuning

Blurry picture: Turn the lens manually until it’s sharp.

Executing code

The COM port is used for communication between PC side and Arduino as follows:

PC side:

Use ReadSerialPortWin project:

- Start the program

- Select the COM port

- WAIT pressing start until the Arduino program has been loaded

Arduino side

Download the program via the COM port before step 3 above.

Do NOT use Serial.begin, Serial.println etc. Everything written to Serial is (of course) sent to the PC program

DO NOT use the port monitor in Arduino IDE / Visual Studio. This will lock the COM port, and then the PC app cannot read it.

Alternative solutions:

NOTE: There may be need for an external oscillator. However, I suspect that this is only needed if we need video. If that’s the case, have a look at this video: https://www.youtube.com/watch?v=gp0FxbEmRSw

This link seems pretty solid, but also use an oscillator: http://www.arducam.com/camera-modules/0-3mp-ov7670/

Converting RAW to image

HERTIL

Installing OV7670 WITH FIFO (=AL422) camera

See also section on OV7670 with FIFO

Links:

https://github.com/ArduCAM/Arduino

http://www.arducam.com/category/introduction/ (I have not used the hardware – only the SW)

http://forum.arduino.cc/index.php?topic=159557.780 
Post #774 has some interesting pin connections to a wifi shield (not the one I have, though).
Post #754 seems interesting

We will in any case need to convert from 5V to 3.3V, and this is shown in this link.
However, the link has some notes stating that it’s the wrong camera type. In contrast to this, the picture shows the right model, and other notes states that it works great. So give it a try. 
http://www.elecfreaks.com/projects/how-to-use-ov7670-camera-module-with-arduino/

Basic “is it alive” setup

With this setup, it’s possible to communicate to the OV7670 (i.e. setting the registers etc.). It is NOT possible to retrieve picture data (no data lines are connected).

Note: The Arduino WeMOS ESP-12 D1 board uses 3.3V, i.e. pull-ups are not necessary between D1 board and OV7670. This is NOT the case for an Arduino UNO board (here, pull-ups must be used).

| ESP8266 D1 | OV7670 FIFO |  |
| --- | --- | --- |
| On-board pin name | On-board pin name |  |
| 3.3V | 3.3V |  |
| GND | GND |  |
| D15/SCL/D3 | SIOC |  |
| D14/SDA/D4 | SIOD |  |
| GND | PWDN |  |
| 3.3V | RST |  |

Sketch supporting this test: OV7670FIFO_simple_communication.ino

Wiring OV7670 FIFO

| ESP8266 D1 |  |  |  |  |  | OV7670 FIFO |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name on board | Pin number (pin1=RX) | PIN pins_arduino.h & common.h | PIN Symbol inherited from pins_arduino.h & common.h in ov7670fifo.c | PIN Symbol value in ov7670fifo.c | PIN Nbr new sw | Name on board | Purpose | In or out |
| 3.3V |  |  |  |  |  | 3.3V |  |  |
| GND |  |  |  |  |  | GND |  |  |
| RX<-D0 | 1 | 3 |  |  |  |  |  |  |
| TX->D1 | 2 |  |  |  |  |  |  |  |
| D2 | 3 |  | DO0 = D2 | 16 |  |  |  |  |
| D15/SCL/D3 | 4 | 5 | DO7 = D3 | 5 | 17 | SIOC | SCCB i/f ctrl clock | IN |
| D14/SDA/D4 | 5 | 4 | DO4 = D4 | 4 | 18 | SIOD | SCCB i/f serial data | IN/OUT |
| D13/SCK/D5 | 6 | 14 | DO3 = D13 DO5 = D5 | 14 (/ 23) | 27 | RRST (blue / old:black) | Read pointer reset terminal |  |
| D12/MISO/D6 | 7 | 12 | DO2 = D12 DO6 = D6 | 12 (/ 25) | 28 | WRST (pink / old:white) | Write pointer reset terminal |  |
| D11/MOSI/D7 | 8 | 13 | DO1 = D11 | 13 |  | n/c |  |  |
| D8 | 9 |  |  |  |  | RCK | FIFO memory read clock control |  |
| TX1/D9 | 10 |  |  |  |  | WR | FIFO write control (1=write, 0=no write) | IN |
| D10/SS | 11 | 15 | SS | 15 |  |  |  |  |
| D11/MOSI | 12 |  | MOSI_PIN | 13 |  |  |  |  |
| D12/MISO | 13 |  | SPI_SCK_PIN | 14 |  |  |  |  |
| D13/SCK | 14 |  |  |  |  |  |  |  |
| GND | 15 |  |  |  |  | OE (green / old:green) | FIFO off control | IN |
| (blank) | 16 |  |  |  |  |  |  |  |
| D14/SCM | 17 |  |  |  |  |  |  |  |
| D15/SCL | 18 |  |  |  |  |  |  |  |
| SS |  | 15 |  |  |  |  |  |  |
|  |  |  |  |  |  | HREF | Line sync | OUT |
|  |  |  |  | 22 |  | VSYNC | Frame sync | OUT |
| GND |  |  |  |  |  | PWDN | Power down | IN |
|  |  |  |  | 35 |  | DO0 |  |  |
| 3.3V |  |  |  | 26 |  | RST | Reset | IN |
| D6 | 7 |  | DO2/DO6 | 12 |  |  |  |  |
| D7 | 8 |  | DO1 | 13 |  |  |  |  |
| D8 |  |  |  |  |  |  |  |  |

The pins on OV7670 are explained here: http://www.electrodragon.com/w/OV7670_Module

Relations to Arduino UNO pins: http://www.instructables.com/id/Programming-the-WeMos-Using-Arduino-SoftwareIDE/?ALLSTEPS

Tuning

Blurry picture: Turn the lens manually until it’s sharp.

Executing code

The COM port is used for communication between PC side and Arduino as follows:

Use this setup:

PC side:

Use ReadSerialPortWin project:

- Start the program

- Select the COM port

- WAIT pressing start until the Arduino program has been loaded

Arduino side

Download the program via the COM port before step 3 above.

Do NOT use Serial.begin, Serial.println etc. Everything written to Serial is (of course) sent to the PC program

DO NOT use the port monitor in Arduino IDE / Visual Studio. This will lock the COM port, and then the PC app cannot read it.

Alternative solutions:

NOTE: There may be need for an external oscillator. However, I suspect that this is only needed if we need video. If that’s the case, have a look at this video: https://www.youtube.com/watch?v=gp0FxbEmRSw

This link seems pretty solid, but also use an oscillator: http://www.arducam.com/camera-modules/0-3mp-ov7670/

Converting RAW to image

HERTIL

Installing wifi shield ESP-01 ESP-8266

This one works: http://www.esp8266.com/wiki/doku.php?id=getting-started-with-the-esp8266 
Tricks:

GPI0 control ESP’s boot mode. Low => flash mode, high => normal boot.

Use external 3.3V power supply.

Use 3.3/5.0 V bi-logic converter between Arduino and ESP.

Out of the box BAUD rate is 115200.

Arduino’s SoftSerial is max 57600 (on receive. It can transmit with 115200).

Default set up as AP with SSID is AI-THINKER_0956F8 (last 6 is MAC addr).

Key Arduino sketches:

Setup_wifi: has all the setup needed

TEST_WIFI is the most basic script. It only sets up communication. Default baud is 115200.

Baud rate. Get it right in all 3 places (ESP, Arduino, and terminal window):

Initialize all 3 with the current BAUD (probably 115200)

Run AT command AT+IPR=57600

Change BAUD in the Arduino script and rerun

Change Baud in the terminal window

Restart:

After power cycle, the ESP will reset to 115200 BAUD. To re-establish do exactly:

Turn off both Arduino and ESP

Wait a while

Turn on ESP

Set BAUD correct in Arduino Sketch and monitor (to 115200)

Reset ESP again

Make sure there’s some kind of communication.

Write new BAUD (57600) to ESP

Set BAUD to 57600 in Arduino Sketch and monitor (to 115200)

How-to: http://shin-ajaran.blogspot.dk/2014/12/noobs-guide-to-esp8266-with-arduino.html

NOTE: SSID is AI-THINKER_0956F8 (last 6 is MAC).

http://www.instructables.com/id/noobs-guide-to-ESP8266-with-Arduino-Mega-2560-or-U/  
               (or: http://dalpix.com/blog/connecting-your-arduino-wifi-esp-8266-module )

Ref doc: http://www.pighixxx.com/test/wp-content/uploads/2014/12/ESP8266Ref.pdf

Wiring

Booting

Note that boot BAUD rate is 74880 !!!!! 
non-marked version

Boot read-out:

ets Jan  8 2013,rst cause:2, boot mode:(3,0)

load 0x40100000, len 1856, room 16

tail 0

chksum 0x63

load 0x3ffe8000, len 776, room 8

tail 0

chksum 0x02

load 0x3ffe8310, len 552, room 8

tail 0

chksum 0x79

csum 0x79

2nd boot version : 1.5

SPI Speed      : 40MHz

SPI Mode       : DIO

SPI Flash Size & Map: 8Mbit(512KB+512KB)

jump to run user1 @ 1000

n-á

SD+P×*Õîj­(

FROM TOOL:

flash vendor:

E0h : N/A

flash devID:

4014h

QUAD;8Mbit

crystal:

26 Mhz

Flashing ESP

Flash using Python tool (esptool.py) on Windows 10 (x64)

Install version 2.7 Python: https://docs.python.org/3/using/windows.html

Install GIT

git clone https://github.com/themadinventor/esptool.git

Install PySerial from normal command prompt (http://pythonhosted.org/pyserial/pyserial.html#installation ):

I:\DEV\Arduino\ESP flasher tool\pyserial-3.1.1>py setup.py install

Flashing:

https://learn.adafruit.com/building-and-running-micropython-on-the-esp8266/flash-firmware

Copy bin file (e.g. “v2.0 AT Firmware(ESP).bin”) to esptool folder.

cd I:\DEV\Arduino\GIT\esptool

py esptool.py -p COM7 --baud 460800 write_flash --flash_size=8m 0 "v2.0 AT Firmware(ESP).bin"

py esptool.py -p COM7 --baud 9600 write_flash -fm dio -fs 8m 0 blank_1MB.bin

(Note: I had to correct the esptool.py. A lot of “print” without brackets. Seems not to work in this Python version, i.e. print ‘xx’ => print(‘xx’)

When you're ready to flash the firmware, first put the ESP8266 into its programming mode.  With the Huzzah ESP breakout do this by holding the GPIO0 button and pressing the reset button, then releasing reset and finally releasing GPIO0.  With a bare ESP8266 breakout you'll want to follow instructions like these to manually pull the GPIO0 pin low and reset the board.

Other flash tools

Download flash tool: http://www.electrodragon.com/w/Category:ESP8266_Firmware_and_SDK#Default_Firmware_.28ESP-12F.2C_and_the_series.29  (password is “electrodragon”)

Run ESP_DOWNLOAD_TOOL_V2.4.exe

Download BIN from https://github.com/espressif/ESP8266_AT

Black version:

ets Jan  8 2013,rst cause:2, boot mode:(3,7)

load 0x40100000, len 612, room 16

tail 4

chksum 0x12

load 0x3ffe8000, len 788, room 4

tail 0

chksum 0x50

load 0x3ffe8314, len 264, room 8

tail 0

chksum 0x4a

csum 0x4a

2nd boot version : 1.1

SPI Speed      : 40MHz

SPI Mode       : QIO

SPI Flash Size : 4Mbit

jump to run user1

user code done

NOTE: Unplug ESP completely and re-insert immediately before flashing. Power off + power on is not enough.

Black cross version

ets Jan  8 2013,rst cause:1, boot mode:(3,7)

load 0x40100000, len 612, room 16

tail 4

chksum 0x12

load 0x3ffe8000, len 788, room 4

tail 0

chksum 0x50

load 0x3ffe8314, len 264, room 8

tail 0

chksum 0x4a

csum 0x4a

2nd boot version : 1.1

SPI Speed      : 40MHz

SPI Mode       : DIO

SPI Flash Size : 16Mbit

jump to run user1

Fatal exception (28):

epc1=0x40001800, epc2=0x00000000, epc3=0x00000000, excvaddr=0x14a1894c, depc=0x00000000

Best result so far:

Connect ESP and FTDI tool as shown here: http://www.martyncurrey.com/arduino-esp8266/

Start NODEMCU flash tool “ESP8266Flasher.exe”

Set 5/3.3 switch to 3.3

Connect FTDI via USB

Note in the flash tool, which COM port shows up. Maybe COM7

To enter flash mode on ESP:

Set GPIO to GND (and keep it there)

“GPIO0- > GND ( Only for when loading the firmware because this pin is what decides if to boot in firmware load mode or regular mode)”

Set RST to GND

Set RST floating (or high)

Now chip is in flash mode

- Press Flash

- To exit flash mode and transmit normally

Set GPIO to 3.3V (and keep it there)

Set RST to GND

Set RST floating (or high)

Now chip is in flash mode

I obtained the flasher software from:

https://github.com/nodemcu/nodemcu-flasher

and the latest firmware from:

https://github.com/nodemcu/nodemcu-firmware

ESP8266 Arduino board (ESP12-E Arduino = D1)

Install drivers and sw

Install board on Arduino (I have a D1, not a D1R1):

Install the “CH340 USB to UART driver” (ch341ser.exe) found on https://www.wemos.cc/downloads

Install by following this: https://www.wemos.cc/tutorial/get-started-arduino.html , i.e.:

Cd J:\Programmer\Arduino\hardware  (install folder of Arduino)

git clone https://github.com/esp8266/Arduino.git esp8266

cd esp8266/tools

python get.py

Restart Arduino IDE

Compile a sketch and upload (this takes quite a while, easily 30 minutes. Have patience)

Change the default sketchbook location to the place where you want your source code:

  - File => Setting => “I:\DEV\Arduino”

Install the Visual Micro plug-in (“Visual.Micro.Arduino.Studio.vsix”) on Visual Studio to integrate the Arduino IDE into VS. (I don’t know if it works to install Visual Micro before installing the ESP-libraries. I had some problems, so I had to de-install and start from scratch).

Install the Arduino ESP8266 board: https://github.com/esp8266/Arduino#installing-with-boards-manager:

- Start Arduino and open Preferences window.

- Enter http://arduino.esp8266.com/stable/package_esp8266com_index.json into Additional Board Manager URLs field. You can add multiple URLs, separating them with commas.

- Open Boards Manager from Tools > Board menu and install esp8266 by ESP8266 Community platform (and don't forget to select your ESP8266 board from Tools > Board menu after installation).

- Restart Arduino IDE

- Rebuild the simple sketch

- (after this, the IDE woulnd’t start at all. I then delete the hardware/esp8266 folder completely (well – moved it). And then it worked (!?)

Attach board to USB and recompile and upload.

Other useful links:

An alternative installation method plus some info about the board: http://www.instructables.com/id/Programming-the-WeMos-Using-Arduino-SoftwareIDE/?ALLSTEPS.

Barcode reader programming

Test programs for development

29/11-16: sending messages to IOT hub from Arduino using Visual Studio:

/DEV/Arduino/src/CommandcenterVS

Gives warning “WARNING: Missing unmatched library header: time.h”. This is OK.

1/12-16: Retrieving a file via http (on windows):

- DEV/GIT/simplecurl

Sending a file over wifi

Check:

https://github.com/itead/ITEADLIB_Arduino_WeeESP8266/tree/master/examples

https://github.com/itead/ITEADLIB_Arduino_WeeESP8266

http://stackoverflow.com/questions/12251551/send-arduino-data-via-a-wi-fi-shield-to-a-specific-ip-address-on-the-lan

Additional shopping:

CD4050 IC (https://www.elextra.dk/main.aspx?page=article&artno=H15876 , 12,00 DKK)

Visual Studio setup for Arduino

Use the free plugin from http://www.visualmicro.com . It includes a (non free) debugger.

NOTE : Do not use the Windows Store version (i.e. the Windows App) of the Arduino IDE. 
Instead use the IDE for download for Windows. The Arduino IDE should be installed first.
I also installed the additional WeMo boards first on the Arduino IDE.

Install following http://www.visualmicro.com/page/User-Guide.aspx?doc=Getting-started.html 
File : Visual.Micro.Arduino.Studio.vsix

Running and debugging

The serial output setup

Restart by either pressing reset hardware button or the green « Connect » button on the IDE serial output window..

Install the right VS extensions :

https://msdn.microsoft.com/en-us/magazine/mt694088.aspx

Visual C++ for Linux Development

Visual C++ for IoT Development (NOT INSTALLED. Requires android setup. Is that really necessary?)

Azure IoT Hub Connected Service

Java extension (for the com-reader)

Important note

.ino files must ONLY contain alphanumeric characters. No underscores, no spaces, no nothing. You will get errors like this:

Compiling 'Multi_Starter2_jmd_ports2-9_VS' for 'Arduino Uno'
core.a(main.cpp.o)*:In function `main'

http://www.visualmicro.com/forums/YaBB.pl?num=1416749264

In case of errors

Class unknown class VisualMicroDebug

There are breakpoints somewhere inthe code. Disable them and recompile. (http://www.visualmicro.com/forums/YaBB.pl?action=print;num=1416846955 )

esptool

For the ESP8266 (« D1 ») board to work in case of a problem with « esptool » or 
 «exec: "J:\\Programmer\\Arduino\\hardware\\esp8266com\\esp8266/tools/xtensa-lx106-elf/bin/xtensa-lx106-elf-g++": file does not exist
Error compiling for board WeMos D1(Retired). »  close Visual Studio and follow this CAREFULLY: 
http://www.instructables.com/id/Portable-installation-guide-of-Arduino-IDE-v165-fo/?ALLSTEPS

Hereafter, restart Visual Studio.

And then:  pip install esptool

Path variables

In my case the following environment variables did not exist, so I created them. Check that the paths exists.

set INCLUDE=j:\Programmer\Microsoft Visual Studio 14.0\VC\INCLUDE;j:\Programmer\Microsoft Visual Studio 14.0\VC\ATLMFC\INCLUDE;C:\Program Files (x86)\Windows Kits\10\Include\10.0.10240.0\ucrt;C:\Program Files (x86)\Windows Kits\NETFXSDK\4.6\include\um;

set LIB=j:\Programmer\Microsoft Visual Studio 14.0\VC\LIB\amd64;j:\Programmer\Microsoft Visual Studio 14.0\VC\ATLMFC\LIB\amd64;C:\Program Files (x86)\Windows Kits\NETFXSDK\4.6\lib\um\x64;

set LIBPATH=C:\Windows\Microsoft.NET\Framework64\v4.0.30319;j:\Programmer\Microsoft Visual Studio 14.0\VC\LIB\amd64;j:\Programmer\Microsoft Visual Studio 14.0\VC\ATLMFC\LIB\amd64;C:\Program Files (x86)\Windows Kits\10\UnionMetadata;C:\Program Files (x86)\Windows Kits\10\References;C:\Program Files (x86)\Windows Kits\10\References\Windows.Foundation.UniversalApiContract\1.0.0.0;C:\Program Files (x86)\Windows Kits\10\References\Windows.Foundation.FoundationContract\1.0.0.0;C:\Program Files (x86)\Windows Kits\10\References\indows.Networking.Connectivity.WwanContract\1.0.0.0;

cmd.exe /k ""C:\Program Files (x86)\Microsoft Visual Studio 14.0\VC\vcvarsall.bat"" amd64

cmd.exe /k ""C:\Program Files (x86)\Microsoft Visual Studio 14.0\VC\vcvarsall.bat"" x64

cmd.exe /k ""C:\Program Files (x86)\Microsoft Visual Studio 14.0\VC\vcvarsall.bat"" x86

Install curl library:

Follow http://www.cplusplus.com/forum/windows/172335/ CAREFULLY. 
I did as follows:

Make sure all environment variables are set as above (“Path variables”)

Install cmake for windows from https://cmake.org/download/ (not sure if this is actually needed afterall)

Install Visual Studio Command prompt if not already existing: http://stackoverflow.com/questions/21476588/where-is-developer-command-prompt-for-vs2013 (exchanging 2013 with 2015)

Follow I:\DEV\GIT\GitRepo\curl\winbuild\BUILD.WINDOWS.txt :

From VS command prompt (NOT regular command prompt):

Cd I:\DEV\GIT\GitRepo\curl\winbuild

nmake /f Makefile.vc mode=dll VC=14

cd I:\DEV\GIT\GitRepo\curl\src

cmake -G "Visual Studio 14 2015 Win64"

Set project to release mode (not debug) in Build => Confuragtion manager

Add the paths to eachVS project usding curl:

VC++ Directories => Include directories:

I:\DEV\GIT\GitRepo\curl\curl-master\builds\libcurl-vc14-x86-release-dll-ipv6-sspi-winssl\include\curl

VC++ Direcotries => Library directories:

I:\DEV\GIT\GitRepo\curl\curl-master\builds\libcurl-vc14-x86-release-dll-ipv6-sspi-winssl\lib

Linker => Input => additional dependencies:

libcurl.lib

Copy the file I:\DEV\GIT\GitRepo\curl\curl-master\builds\libcurl-vc14-x86-release-dll-ipv6-sspi-winssl\bin\libcurl.dll to the project source code folder (I:\DEV\GIT\simplecurl\simplecurl).

Add libcurl.lib at the end of this line:

Copy the file I:\DEV\GIT\GitRepo\curl\curl-master\builds\libcurl-vc14-x86-release-dll-ipv6-sspi-winssl\bin\libcurl.dll to the project source code folder (I:\DEV\GIT\simplecurl\simplecurl).

The VS project simplecurl gets a file from an external site, downloads and stores it locally.

Java (not used)

NOT used because the ReadSerial program didn’t work. Cause: the latest JDK (v8) does not support the javax.comm library.

Remove all Java and Java Dev Kit versions and updates. Start from a clean sheet.

Install latest Java SDK 32-bit version (there’s no working javax.comm for 64-bit) http://stackoverflow.com/questions/3959743/javax-comm-api-on-64-bit-windows

Install Java Visual Studio plugin

Install javax.comm API’s

Note: Not supported on any windows platform anymore

install

: https://circuitnegma.wordpress.com/2007/02/07/how-to-install-the-java-communications-api-in-a-windows-environment/

Copy 'win32com.dll' to JDK_HOME\jre\bin. (C:\Program Files (x86)\Java\jdk1.8.0_121\jre\bin)

Copy 'javax.comm.properties'to to JDK_HOME\jre\lib.

Copy 'comm.jar'to to JDK_HOME\jre\lib